# NEEDS UPDATE

# Full parts list

## Coil stack

- Bottom CuCr Disk x1

  - Machined by 3ERP out of C182 (CuCr alloy)

  - Immediately before assembly, cleaned with hot acetic acid

- Copper discs (normal) x30

  - Waterjet in house

  - From 0.04" thick 101 copper sheet, PN 89675K736

  - Sanded with grits 220, 400, 600, 1200, 2000

  - Immediately before assembly, cleaned with hot acetic acid

- Copper disc (top) x1

  - Waterjet in house

  - From 0.04" thick 101 copper sheet, PN 89675K736

  - Sanded with grits 220, 400, 600, 1200, 2000

  - Immediately before assembly, cleaned with hot acetic acid

- Copper connecting wedges x31

  - Waterjet in house

  - Leave tabs so they don’t fall off

  - Sandwich between two sheets of aluminum (89015K37) so they don't completely warp, then use the thickness of the resulting sandwich in the waterjet settings

  - File off the tabs with the file side to side, i.e. along the axis of the tab, slowly and gently.

  - Made from 9014K174, the shim stock is important because it has a much better thickness tolerance.

  - Sanded with grits 600, 1200, 2000. Sand lightly so you don't wear down the thickness too much.

  - Immediately before assembly, cleaned with hot acetic acid

- Polyester spacers x31

  - Laser cut in house

  - From 0.01" thick brown shim stock, PN 9513K119

  - Use the PETG setting, 0.01" thick on the laser cutter

- Polyester spacer top x1

  - Laser cut in housee

  - From 0.01" thick brown shim stock, PN 9513K119

  - Use the PETG setting, 0.01" thick on the laser cutter

- Silicone banana gaskets  x94

  - Laser cut in house

  - From 0.02" thick high-temperature silicon rubber strips, durometer 40A (medium soft), PN 5344N113

  - Use the silicone setting, 0.02" thick on the laser cutter

  - Note that the silicone pieces tend to fly away due to exhaust wind!

  - After cutting, sonicate in water then rinse in a strainer

- Silicone circle gaskets  x64

  - Laser cut in house

  - From 0.02" thick high-temperature silicon rubber strips, durometer 40A (medium soft), PN 5344N113

  - Use the silicone setting, 0.02" thick on the laser cutter

  - Note that the silicone pieces tend to fly away due to exhaust wind!

  - After cutting, sonicate in water then rinse in a strainer

## Compression screw system

- Stainless steel compression screw x7

  - Machined by 3ERP out of 316 stainless steel

- CuCr Screw x1

  - Machined by 3ERP out of C182 (CuCr alloy)

  - Path: RevisedManufacturing20231107/MachineParts/Steel-Compression-Rod.step

  - Immediately before assembly, cleaned with hot acetic acid. I think sanding will only hurt, the surface finish from 3ERP rocks

  - Note for future: make the groove in the top a bit deeper and/or make something stick off the top that you can grab with a wrench, the slot I milled this time is a bit thin and wore out quick under stress.

- O-rings x8

  - Chemical-Resistant Viton® Fluoroelastomer O-Ring 1/8 Fractional Width, Dash Number 202, PN 9464K118

- Top washers (many, at least 24)

  - 316 Stainless Steel Washer for 1/4" Screw Size, 0.281" ID, 0.625" OD, PN 90107A029

- Nuts for top of steel screws

  - 316 Stainless Steel Hex Nut, 1/4"-28 Thread Size, 94804A305

- Nut for CuCr screw x1

  - 316 Stainless Steel Thin Hex Nut, Super-Corrosion-Resistant, 5/16"-24 Thread Size, 94805A213

- Washers for CuCr screw (at least 3-4)

  - 316 Stainless Steel Washer for 5/16" Screw Size, 0.344" ID, 0.75" OD, 90107A030

## 3D printed piece & water system

- 3D printed piece

  - Printed in SLA "High Temperature Resin" by a company we found through CraftCloud, called 3DPnxt

  - OR maybe printed in SLA "Surgical Guide Resin" this is the recommendation from Shaozhen!

- NPT tap

- Vibra-TITE 481 Plastic Thread Sealant 70 mL

- Plastic NPT to push-to-connect fitting

- Vacuum chamber screws

  - Ours are probably 1/4-20 or 1/4-28

## Bus bar system

- Bus bar top

  - C101

  - Machined by 3ERP

  - Immediately before assembly, cleaned with hot acetic acid

- Bus bar bottom

  - C101

  - Machined by 3ERP

  - Immediately before assembly, cleaned with hot acetic acid

- Top bus bar bracket

  - Machined by 3ERP

- CuCr screw

  - Machined by 3ERP

- Plastic separator  x1

  - Laser cut in house

  - 0.01" brown plastic shim stock

- O ring x1

  - Chemical-Resistant Viton® Fluoroelastomer O-Ring, 3/32 Fractional Width, Dash Number 108

  - 9464K113

  - Note this is DIFFERENT than the other ones

- Tube fittings x2

  - Push-to-Connect Tube Fitting for Air and Water, Nickel-Plated Adapter, for 1/4" Tube OD, 1/16 NPT Male

  - 7880T123

- Countersunk 2-56 nylon screws, 3/4" long, x2

  - PN 92929A120

- Simple 2-56 nylon screws, 1" long, x6

  - 94613A086

- Bus bar mounting screws: 6-32, 7/8" long, x6

  - 92185A152

- Top bus bar bracket screws x2

  - 1-1/8" long, 316 stainless steel

  - PN 92185A154

- Bottom bus bar bracket screws x2

  - 1-5/8" long, 316 stainless steel

  - PN 92185A174

- Lug screws

  - 1/4"-20 Thread Size, 1/2" Long

  - PN 93190A537

- Lug washers

  - 1/4" washers

## Assembly props

- Steel dowels 1/4" x 3"

  - PN 98381A554

- Clamp jaws, top and bottom

  - Make out of random scrap from machine shop

  - Or ask DeMille lab to borrow?